# ISA Bus Technical Summary

## Table of Contents

### 1.0 ISA Overview

### 2.0 ISA Documents

2.1 ISA Specifications
2.2 ISA Books

### 3.0 ISA Signal Descriptions

### 4.0 ISA Timing Diagrams

### 5.0 ISA Signal Usage

### 6.0 ISA Connector Pinout

## 1.0 ISA Overview

The Industry Standard Architecture, or ISA, bus originated in the early 1980s at an IBM development lab in Boca Raton, Florida. The original IBM Personal Computer introduced in 1981 included the 8-bit subset of the ISA bus. In 1984, IBM introduced the PC-AT which was the first full 16-bit implementation of the ISA bus.

The "AT bus", as IBM originally called it, was first documented in an IBM publication called the PC-AT Technical Reference. The Technical Reference included schematics and BIOS listings that made it easy for other companies like Compaq to produce IBM compatible clones. The companies producing IBM compatibles could not use the "AT bus" name however since IBM had protected it with a trademark. In response, the industry coined "ISA" as a new name for the the bus that was eventually adopted by everyone including IBM.

Although the PC-AT Technical Reference included detailed schematics and BIOS listings, it did not include the rigorous timings, rules, and other requirements that would make it a good bus specification. As a result, the various implementations of ISA were not always compatible with each other. Over time various ISA bus specifications were produced in an attempt to alleviate the compatibility problems. But unfortunately these specifications did not always agree with each other, so no single specification for the ISA bus was ever developed.

## 2.0 ISA Documents

### 2.1 ISA Specifications

Several documents that include specifications for the ISA bus are as follows:

- EISA Specification, Version 3.12 - This document includes specifications for ISA as well as the "Extended Industry Standard Architecture" that defined a 32-bit extension to the ISA bus. At last report, this document could be ordered for a fee ($132) from BCPR Services, Inc., P.O. Box 11137, Spring, TX 77391-1137, (713) 251-4770, FAX: (713) 251-4832.

- IEEE Draft Standard P996 - This document describes the mechanical and electrical specifications for standard PC-style systems. It may be ordered for a fee from IEEE at http://standards.ieee.org/.

- PS/2 Technical Reference - AT Bus Systems - This document from IBM includes signal definitions and timing diagrams for the ISA bus used in some of IBM's PS/2 line of computers. It can be ordered from IBM at http://www.ibmlink.ibm.com. Search IBM's PubCatalog for document number S85F-1646.

### 2.2 ISA Books

Two books that provide good descriptions of the ISA bus are:

- ISA & EISA Theory and Operation, by Edward Solari. (Annabooks) (ISBN 0-929392-15-9)

- ISA System Architecture, by Don Anderson and Tom Shanley (MindShare) (ISBN 0-201-40996-8)

## 3.0 ISA Signal Descriptions

SA19 to SA0

System Address bits 19:0 are used to address memory and I/O devices within the system. These signals may be used along with LA23 to LA17 to address up to 16 megabytes of memory. Only the lower 16 bits are used during I/O operations to address up to 64K I/O locations. SA19 is the most significant bit. SA0 is the least significant bit. These signals are gated on the system bus when BALE is high and are latched on the falling edge of BALE. They remain valid throughout a read or write command. These signals are normally driven by the system microprocessor or DMA controller, but may also be driven by a bus master on an ISA board that takes ownership of the bus.

LA23 to LA17

Unlatched Address bits 23:17 are used to address memory within the system. They are used along with SA19 to SA0 to address up to 16 megabytes of memory. These signals are valid when BALE is high. They are "unlatched" and do not stay valid for the entire bus cycle. Decodes of these signals should be latched on the falling edge of BALE.

AEN

Address Enable is used to negate the system microprocessor and other devices from the bus during DMA transfers. When this signal is active the system DMA controller has control of the address, data, and read/write signals. This signal should be included as part of ISA board select decodes to prevent incorrect board selects during DMA cycles.

BALE

Buffered Address Latch Enable is used to latch the LA23 to LA17 signals or decodes of these signals. Addresses are latched on the falling edge of BALE. It is forced high during DMA cycles. When used with AEN, it indicates a valid microprocessor or DMA address.

CLK

System Clock is a freerunning clock typically in the 8MHz to 10MHz range, although its exact frequency is not guaranteed. It is used in some ISA board applications to allow synchronization with the system microprocessor.

SD15 to SD0

System Data serves as the data bus bits for devices on the ISA bus. SD15 is the most significant bit. SD0 is the least significant bits. SD7 to SD0 are used for transfer of data with 8-bit devices. SD15 to SD0 are used for transfer of data with 16-bit devices. 16-bit devices transferring data with 8-bit devices shall convert the transfer into two 8-bit cycles using SD7 to SD0.

-DACK0 to -DACK3 and -DACK5 to -DACK7

DMA Acknowledge 0 to 3 and 5 to 7 are used to acknowledge DMA requests on DRQ0 to DRQ3 and DRQ5 to DRQ7.

DRQ0 to DRQ3 and DRQ5 to DRQ7

DMA Requests are used by ISA boards to request service from the system DMA controller or to request ownership of the bus as a bus master device. These signals may be asserted asynchronously. The requesting device must hold the request signal active until the system board asserts the corresponding DACK signal.

-I/O CH CK

I/O Channel Check signal may be activated by ISA boards to request than an non-maskable interrupt (NMI) be generated to the system microprocessor. It is driven active to indicate a uncorrectable error has been detected.

I/O CH RDY

I/O Channel Ready allow slower ISA boards to lengthen I/O or memory cycles by inserting wait states. This signals normal state is active high (ready). ISA boards drive the signal inactive low (not ready) to insert wait states. Devices using this signal to insert wait states should drive it low immediately after detecting a valid address decode and an active read or write command. The signal is release high when the device is ready to complete the cycle.

-IOR

I/O Read is driven by the owner of the bus and instructs the selected I/O device to drive read data onto the data bus.

-IOW

I/O Write is driven by the owner of the bus and instructs the selected I/O device to capture the write data on the data bus.

IRQ3 to IRQ7 and IRQ9 to IRQ12 and IRQ14 to IRQ15

Interrupt Requests are used to signal the system microprocessor that an ISA board requires attention. An interrupt request is generated when an IRQ line is raised from low to high. The line must be held high until the microprocessor acknowledges the request through its interrupt service routine. These signals are prioritized with IRQ9 to IRQ12 and IRQ14 to IRQ15 having the highest priority (IRQ9 is the highest) and IRQ3 to IRQ 7 have the lowest priority (IRQ7 is the lowest).

-SMEMR

System Memory Read instructs a selected memory device to drive data onto the data bus. It is active only when the memory decode is within the low 1 megabyte of memory space. SMEMR is derived from MEMR and a decode of the low 1 megabyte of memory.

-SMEMW

System Memory Write instructs a selected memory device to store the data currently on the data bus. It is active only when the memory decode is within the low 1 megabyte of memory space. SMEMW is derived from MEMW and a decode of the low 1 megabyte of memory.

-MEMR

Memory Read instructs a selected memory device to drive data onto the data bus. It is active on all memory read cycles.

-MEMW

Memory Write instructs a selected memory device to store the data currently on the data bus. It is active on all memory write cycles.

-REFRESH

Memory Refresh is driven low to indicate a memory refresh operation is in progress.

OSC

Oscillator is a clock with a 70ns period (14.31818 MHz). This signal is not synchronous with the system clock (CLK).

RESET DRV

Reset Drive is driven high to reset or initialize system logic upon power up or subsequent system reset.

TC

Terminal Count provides a pulse to signal a terminal count has been reached on a DMA channel operation.

-MASTER

Master is used by an ISA board along with a DRQ line to gain ownership of the ISA bus. Upon receiving a -DACK a device can pull -MASTER low which will allow it to control the system address, data, and control lines. After -MASTER is low, the device should wait one CLK period before driving the address and data lines, and two clock periods before issuing a read or write command.

-MEM CS16

Memory Chip Select 16 is driven low by a memory slave device to indicate it is capable of performing a 16-bit memory data transfer. This signal is driven from a decode of the LA23 to LA17 address lines.

-I/O CS16

I/O Chip Select 16 is driven low by a I/O slave device to indicate it is capable of performing a 16-bit I/O data transfer. This signal is driven from a decode of the SA15 to SA0 address lines.

-0WS

Zero Wait State is driven low by a bus slave device to indicate it is capable of performing a bus cycle without inserting any additional wait states. To perform a 16-bit memory cycle without wait states, -0WS is derived from an address decode.

-SBHE

System Byte High Enable is driven low to indicate a transfer of data on the high half of the data bus (D15 to D8).

## 4.0 ISA Bus Timing Diagrams

### 8-Bit I/O Bus Cycles

________

BALE      __|        |_________________________________________

_  ______________________________________________  __

SA(15:0)  _><______________________________________________><__

-SBHE

______________                                _______

-IOR/W                  |______________________________|

_____________

SD(7:0)   -------------------------------------<_____________>-

(READ)

__________________________________

SD(7:0)   ----------------<__________________________________>-

(WRITE)

__________________ _ _ _ _ _ _ _ _ _ _ _ _  _________

I/OCHRDY                    |________________________|

### 8-Bit Memory Bus Cycles

_____

BALE      ________|     |______________________________________

_  ________________  ________________________________

LA(23:17) _><________________><________________________________

_______  ________________________________________  __

SA(19:0)  _______><________________________________________><__

______________                                _______

-MEMR/W                 |______________________________|

_____________

SD(7:0)   -------------------------------------<_____________>-

(READ)

__________________________________

SD(7:0)   ----------------<__________________________________>-

(WRITE)

__________________ _ _ _ _ _ _ _ _ _ _ _ _  _________

I/OCHRDY                    |________________________|

### 16-Bit I/O Bus Cycles

________

BALE      ______________|        |_____________________________

_____________  __________________________________  __

SA(15:0)  _____________><__________________________________><__

_________________                                 ___

-IOCS16                    |_______________________________|

_____________________                          ______

-IOR/W                         |________________________|

__________________

SD(15:0)  -----------------------------<__________________>----

(READ)

________________________

SD(15:0)  -----------------------<________________________>----

(WRITE)

_______________________ _ _ _ _ _ _ _ _ _ _ ______

I/OCHRDY                         |___________________|

### 16-Bit Memory Bus Cycles (1 or more Wait States)

______

BALE      _________________|      |____________________________

___  ________________________  ______________________

LA(23:17) ___><________________________><______________________

________________  ________________________________  _

SA(19:0)  ________________><________________________________><_

_______                        ______________________

-MEMCS16         |______________________|

________________________                       ______

-MEMR/W                           |_____________________|

_______________

SD(15:0)  --------------------------------<_______________>----

(READ)

_____________________

SD(15:0)  --------------------------<_____________________>----

(WRITE)

__________________________ _ _ _ _ _ _ _ _ __________

I/OCHRDY                            |_______________|

### 16-Bit Memory Bus Cycles (0 Wait State)

______

BALE      _________________|      |____________________________

___  ________________________  ______________________

LA(23:17) ___><________________________><______________________

________________  _________________________  ________

SA(19:0)  ________________><_________________________><________

_______                        ______________________

-MEMCS16         |______________________|

_________________________      ______________________

-0WS                               |____|

________________________             ________________

-MEMR/W                           |___________|

______

SD(15:0)  --------------------------------<______>-------------

(READ)

____________

SD(15:0)  --------------------------<____________>-------------

(WRITE)

### DMA Read

______________

DRQ(n)    __|              |___________________________________

_______________                            __________

-DACK(n)                 |__________________________|

____________________________________

AEN,BALE  ________|                                    |_______

_______________  ___________________________  _______

SA(15:0)  _______________><___________________________><_______

-SBHE

________________  ________________________  _________

SA(19:16) ________________><________________________><_________

LA(23:17)

____________________                       __________

-MEMR                         |_____________________|

____________

SD(15:0)  -------------------------------<____________>--------

______________________                    ___________

-IOW                            |__________________|

__________

TC        _______________________________|          |__________

________________________        _____________________

I/OCHRDY                          |______|

### DMA Write

______________

DRQ(n)    __|              |___________________________________

_______________                            __________

-DACK(n)                 |__________________________|

____________________________________

AEN,BALE  ________|                                    |_______

_______________  ___________________________  _______

SA(15:0)  _______________><___________________________><_______

-SBHE

________________  ________________________  _________

SA(19:16) ________________><________________________><_________

LA(23:17)

____________________                       __________

-IOR                          |_____________________|

____________

SD(15:0)  -------------------------------<____________>--------

______________________                    ___________

-MEMW                           |__________________|

__________

TC        _______________________________|          |__________

________________________        _____________________

I/OCHRDY                          |______|

### Alternate Bus Master Cycles

___________________________________

DRQ(n)    __|                                   |______________

_______________                            __________

-DACK(n)                 |__________________________|

__________________                            _______

-MASTER                     |__________________________|

__________________                            _______

AEN       ________|         |__________________________|     |_

_____________________________________________________

BALE      ________|                                          |_

________________________  ___________  ______________

SA(19:0)  ________________________><___________><_______________

-SBHE

________________________  ___________  ______________

LA(23:17) ________________________><___________><______________

_____________________________       _________________

-IOR,-IOW                              |_____|

-MEMR,-MEMW

_____

SD(15:0)  -------------------------------<_____>---------------

### Memory Refresh Cycles

_______________                       _______________

-REFRESH                 |_____________________|

_________________  ____________  ____________________

SA(9:0)   _________________><____________><____________________

______________________               ________________

-SMEMR                          |_____________|

_________________________ _ _ _ _ ___________________

I/OCHRDY                           |_______|

## 5.0 ISA Signal Usage

Legend:

I/O = Input and Output

I   = Input

O   = Output

-   = Signal not needed

An I/O shown in parentheses () indicates that the signal is

optional for this devices.

The following table indicates typical signal usage by an ISA system board:

| Signal Name | System Board Usage |  | Signal Name | System Board Usage |
| --- | --- | --- | --- | --- |
| AEN | O |  | -MEM CS16 | I/O |
| BALE | O |  | -MEMR | I/O |
| CLK | O |  | -MEMW | I/O |
| -DACK | O |  | OSC | O |
| DRQ | I |  | -REFRESH | I/O |
| -IO CS16 | I |  | RESET DRV | O |
| -I/O CH CK | I |  | SA | I/O |
| I/O CH RDY | I/O |  | SD | I/O |
| -IOR | I/O |  | -SBHE | I/O |
| -IOW | I/O |  | -SMEMR | I/O |
| IRQ | I |  | -SMEMW | I/O |
| LA | I/O |  | TC | I/O |
| -MASTER | I |  | -0WS | I |

The following table indicates typical signal usage by the various types of ISA expansion boards:

| Signal Name | ISA Bus Master | ISA 16-bit Mem Slave | ISA 16-bit I/O Slave | ISA 8-bit Mem Slave | ISA 8-bit I/O Slave | ISA DMA Device |
| --- | --- | --- | --- | --- | --- | --- |
| AEN | - | - | I | - | I | - |
| BALE | - | I | - | (I) | - | - |
| CLK | (I) | (I) | (I) | (I) | (I) | (I) |
| -DACK | I | - | - | - | - | I |
| DRQ | O | - | - | - | - | O |
| -IO CS16 | I | - | O | - | - | - |
| -I/O CH CK | (O) | (O) | (O) | (O) | (O) | (O) |
| I/O CH RDY | I | (O) | (O) | (O) | (O) | - |
| -IOR | O | - | I | - | I | I |
| -IOW | O | - | I | - | I | I |
| IRQ | (O) | (O) | (O) | (O) | (O) | (O) |
| LA(23:17) | O | I | - | (I) | - | - |
| -MASTER | O | - | - | - | - | - |
| -MEM CS16 | I | 0 | - | - | - | - |
| -MEMR | O | I | - | (I) | - | - |
| -MEMW | O | I | - | (I) | - | - |
| OSC | (I) | (I) | (I) | (I) | (I) | (I) |
| -REFRESH | (O) | I | - | I | - | - |
| RESET DRV | I | I | I | I | I | I |
| SA(16:0) | O | I | I | I | I | - |
| SA(19:17) | - | (I) | - | (I) | - | - |
| SD(7:0) | I/O | I/O | I/O | I/O | I/O | I/O |
| SD(15:8) | I/O | I/O | I/O | - | - | (I/O) |
| -SBHE | O | I | I | - | - | - |
| -SMEMR | - | - | - | I | - | - |
| -SMEMW | - | - | - | I | - | - |
| TC | - | - | - | - | - | (I) |
| -0WS | - | (O) | - | (O) | (O) | - |

## 6.0 ISA Connector Pinout

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| Ground | B1 | A1 | -I/O CH CK |
| RESET DRV | B2 | A2 | SD7 |
| +5 V dc | B3 | A3 | SD6 |
| IRQ 9 | B4 | A4 | SD5 |
| -5 V dc | B5 | A5 | SD4 |
| DRQ2 | B6 | A6 | SD3 |
| -12 V dc | B7 | A7 | SD2 |
| -0WS | B8 | A8 | SD1 |
| +12 V dc | B9 | A9 | SD0 |
| Ground | B10 | A10 | I/O CH RDY |
| -SMEMW | B11 | A11 | AEN |
| -SMEMR | B12 | A12 | SA19 |
| -IOW | B13 | A13 | SA18 |
| -IOR | B14 | A14 | SA17 |
| -DACK3 | B15 | A15 | SA16 |
| DRQ3 | B16 | A16 | SA15 |
| -DACK1 | B17 | A17 | SA14 |
| DRQ1 | B18 | A18 | SA13 |
| -REFRESH | B19 | A19 | SA12 |
| CLK | B20 | A20 | SA11 |
| IRQ7 | B21 | A21 | SA10 |
| IRQ6 | B22 | A22 | SA9 |
| IRQ5 | B23 | A23 | SA8 |
| IRQ4 | B24 | A24 | SA7 |
| IRQ3 | B25 | A25 | SA6 |
| -DACK2 | B26 | A26 | SA5 |
| TC | B27 | A27 | SA4 |
| BALE | B28 | A28 | SA3 |
| +5 V dc | B29 | A29 | SA2 |
| OSC | B30 | A30 | SA1 |
| Ground | B31 | A31 | SA0 |
| Key |  |  |  |
| -MEM CS16 | D1 | C1 | -SBHE |
| -IO CS16 | D2 | C2 | LA23 |
| IRQ10 | D3 | C3 | LA22 |
| IRQ11 | D4 | C4 | LA21 |
| IRQ12 | D5 | C5 | LA20 |
| IRQ15 | D6 | C6 | LA19 |
| IRQ14 | D7 | C7 | LA18 |
| -DACK0 | D8 | C8 | LA17 |
| DRQ0 | D9 | C9 | -MEMR |
| -DACK5 | D10 | C10 | -MEMW |
| DRQ5 | D11 | C11 | SD08 |
| -DACK6 | D12 | C12 | SD09 |
| DRQ6 | D13 | C13 | SD10 |
| -DACK7 | D14 | C14 | SD11 |
| DRQ7 | D15 | C15 | SD12 |
| +5 V dc | D16 | C16 | SD13 |
| -MASTER | D17 | C17 | SD14 |
| Ground | D18 | C18 | SD15 |

Copyright © 1999 TechFest.com. All Rights Reserved..